# BoatLogger TRACKER SYSTEM: PROGRAMMER & TECHNICAL MANUAL

System Core: LilyGo T-A7670E (ESP32) & LAMP Stack Infrastructure
Target Environments: MicroPython / PHP 8.x + MariaDB
System Version: 3.0

# 1. Architectural Overview & Principle of Operation

The Gulliver Tracker operates as an asynchronous, closed-loop telemetry logging and remote instruction downlink platform. The system is split across two core operational environments:

- The Client Terminal Node (main.py): An embedded endpoint running MicroPython that samples geographic vectors (GPS), hardware power levels, I2C environmental sensors, and physical toggles. It manages a Store & Forward telemetry cache and coordinates a prioritized network uplink strategy.

- The Server Infrastructure Backend: A LAMP directory environment mapping permanent database record layers (MariaDB), localized volatile RAM/disk caches (boat_data.json), configuration downlinks (control_buffer.json), and dynamic tracking maps.

## 1.1 Embedded Node Execution Lifecycle

- Dynamic Clock Shifting: The system alters its CPU operating profiles based on cryptographic and mathematical loads. It boosts to 160MHz-240MHz during Wi-Fi WPA2 crypto-handshakes, drops to 40MHz-80MHz during AT-command parsing, and throttles down to 20MHz during hardware standby sleep to conserve battery.

- Store & Forward Memory Shield: If the vessel loses cellular service, the script appends telemetry payloads to an internal flash backlog. When a connection is re-established, a recovery function sequentially dispatches the payloads to the server using a &delayed=1 HTTP flag.

- Memory Corruption Shield: The embedded serial interface is explicitly opened with an expanded reception memory ceiling (rxbuf=2048). This safe-caches local line echoes and multi-line server responses, completely eliminating memory truncation errors.

# 2. Component Specifications: Embedded Node (main.py)

## 2.1 Function Directory & Code Block Analysis

- run_emergency_portal(): Initiated via a rapid physical switch 'wiggle' gesture. Forces the ESP32 to broadcast a standalone Wi-Fi Access Point (Gulliver-Emergency-Net) and serves a raw HTML configuration page directly from internal flash memory to allow offline parameter overrides.

- read_barometric_pressure(): Queries a BMP280 module on the I2C bus (0x76/0x77). Unpacks 24 bytes of factory compensation coefficients and runs a floating-point compensation matrix to calculate atmospheric pressure in hPa.

- calculate_drift(lat1, lng1, lat2, lng2): Bypasses bulky floating-point Haversine conversions by implementing a specialized, low-overhead Equirectangular Flat-Earth projection equation optimized for microprocessors.

- process_server_handshake(): Parses the incoming server response string. Utilizes a strict timestamp evaluation to ensure stale server commands do not overwrite newer configurations established manually via the local Emergency Portal.

## 2.2 Network Communications Engine

Strategy 1: Local Area Wi-Fi Pipeline

- Zombie Socket Purging: To survive rapid-fire uploads (backlog recovery), the HTTP GET requests enforce strict socket closures using headers={'Connection': 'close'} followed by response.close() and gc.collect(). This prevents RAM fragmentation (-202 errors).

Strategy 2: Cellular Ingestion Stack

- The 30-Second Settlement Loop: Modern modems process handshakes asynchronously. The cellular pathway uses a high-frequency polling loop (AT+CGREG?) to actively monitor network registration, breaking out the exact millisecond the tower grants an IP context.

- Length-Bounded Transmission Protocol: The data packet uses strict length-bounded framing (AT+CIPSEND=0, [length]). Trailing execution tokens (chr(26) / Ctrl+Z) are strictly omitted to prevent serial corruption.

# 3. Server Web Repository Configurations

## 3.1 log.php – Primary Data Ingestion Gate API

- Temporal Syncing: Intercepts the URL-encoded atomic time query parameter, explicitly assigns it a UTC timezone object, and converts it to Europe/London local time to handle daylight saving shifts.

- Epoch Translation: Converts standard Unix Epoch timestamps (1970) into MicroPython's required hardware Epoch (2000) by mathematically shifting the timestamp (- 946684800).

- Dynamic Vessel Routing: Checks the incoming device_id string and automatically routes caching data to distinct files (e.g., boat_data_gulliver.json).

## 3.2 index.php – Mission Control Portal

- Push Notification & Web Audio Siren: Monitors the telemetry status flag. If a geofence breach is detected, the UI triggers the HTML5 Web Audio API to synthesize a dual-tone hardware siren and dispatches an OS-level Push Notification.

- Staged Workspace Rendering: Map renders a Solid Circle for the actual geographic boundaries reported by the boat, and a Dashed Amber Circle for operator workspace staging. The staging ring remains locked until the boat echoes the new parameters back in telemetry.

- Cryptographic Access Flow: Sensitive configurations are protected by an asynchronous challenge gate cross-referenced against a pre-calculated SHA-256 hash before writing to control_buffer.json.

## 3.3 Diagnostic & Support Utilities

- viewlogs.php: Parses raw JSON history APIs to generate hardware performance canvases using Chart.js. Runs geospatial math to calculate Time-in-Motion and Average Speed metrics.

- diagnoseUploads.php: Evaluates timestamps across the MariaDB database to identify network dropout gaps >15 minutes. Generates a logarithmic histogram to visualize cellular tower reliability.

- get_history.php: Queries MariaDB for older location points, calculates chronological age metrics, and passes them to Leaflet to draw a historical track line.

# 4. Diagnostics & System Troubleshooting Matrix

| Symptom / Log Error | Probable Cause | Remedial Action |
| --- | --- | --- |
| +CIPOPEN: 0,1 | Target server domain cannot be reached. VPN overwriting domain token or DuckDNS fault. | Reset DuckDNS account token. Update duck.sh script. |
| Transmission fails with 358-byte cutoff | Ring buffer overflowing because default 256-byte serial allocation is full. | Confirm rxbuf=2048 is explicitly set in machine.UART constructor. |
| +CIPERROR: 4 right after CIPSEND | A trailing chr(26) character was sent after a length-bounded data transfer. | Remove all uart.write(chr(26)) commands. Let modem auto-send. |
| Session payload drop error: -202 | RAM fragmented due to lingering ghost HTTP sockets. | Ensure headers={'Connection': 'close'} and response.close() execute. |